# Packet Tracer - Propagate a Default Route in OSPFv2 (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Addressing Table

| Device | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 172.16.1.1 | 255.255.255.0 | N/A |
| R1 | S0/0/0 | 172.16.3.1 | 255.255.255.252 | N/A |
| R1 | S0/0/1 | 192.168.10.5 | 255.255.255.252 | N/A |
| R2 | G0/0 | 172.16.2.1 | 255.255.255.0 | N/A |
| R2 | S0/0/0 | 172.16.3.2 | 255.255.255.252 | N/A |
| R2 | S0/0/1 | 192.168.10.9 | 255.255.255.252 | N/A |
| R2 | S0/1/0 | 209.165.200.225 | 255.255.255.224 | N/A |
| R3 | G0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
| R3 | S0/0/0 | 192.168.10.6 | 255.255.255.252 | N/A |
| R3 | S0/0/1 | 192.168.10.10 | 255.255.255.252 | N/A |
| PC1 | NIC | 172.16.1.2 | 255.255.255.0 | 172.16.1.1 |
| PC2 | NIC | 172.16.2.2 | 255.255.255.0 | 172.16.2.1 |
| PC3 | NIC | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| Web Server | NIC | 64.100.1.2 | 255.255.255.0 | 64.100.1.1 |

# Objectives

Part 1: Propagate a Default Route

Part 2: Verify Connectivity

# Background

In this activity, you will configure an IPv4 default route to the Internet and propagate that default route to other OSPF routers. You will then verify the default route is in downstream routing tables and that hosts can now access a web server on the Internet.

# Instructions

## Propagate a Default Route

### Test connectivity to the Web Server

      - From PC1, PC2, and PC3, attempt to ping the Web Server IP address, 64.100.1.2.

#### Questions:

Were any of the pings successful?

Type your answers here.

No.

What message did you receive, and which device issued the message?

Type your answers here.

Destination unreachable from R2.

      - Examine the routing tables on routers R1, R2, and R3.

Open configuration window

#### Question:

What statement is present in the routing tables that indicates that the pings to the Web Server will fail?

Type your answers here.

Gateway of last resort is not set

### Configure a default route on R2.

Configure R2 with a directly attached default route to the Internet.

R2(config)# ip route 0.0.0.0 0.0.0.0 Serial0/1/0

Note: Router will give a warning that if this interface is not a point-to-point connection, it may impact performance. You can ignore this warning because it is a point-to-point connection.

### Propagate the route in OSPF.

Configure OSPF to propagate the default route in OSPF routing updates.

R2(config)# router ospf 1

R2(config-router)# default-information originate

### Examine the routing tables on R1 and R3.

Examine the routing tables of R1 and R3 to verify that the route has been propagated.

R1> show ip route

<output omitted>

Gateway of last resort is 172.16.3.2 to network 0.0.0.0

<output omitted>

O*E2 0.0.0.0/0 [110/1] via 172.16.3.2, 00:00:08, Serial0/0/0

!-------------------

R3> show ip route

<output omitted>

Gateway of last resort is 192.168.10.9 to network 0.0.0.0

<output omitted>

O*E2 0.0.0.0/0 [110/1] via 192.168.10.9, 00:08:15, Serial0/0/1

Close configuration window

## Verify Connectivity

Verify that PC1, PC2, and PC3 can ping the web server.

End of document

# Answer script

# Router R2

enable

config terminal

ip route 0.0.0.0 0.0.0.0 Serial0/1/0

router ospf 1

default-information originate