Part 1 - Step 2:

What is the IP address of the computer?
 What is the subnet mask of the computer?
 What is the default gateway of the computer?

- IP address: 192.168.0.101

- Subnet mask: 255.255.255.0

- Default gateway: 192.168.0.1

Part 1 - Step 3:

Is the IP address for Admin within this range? Is it expected? Explain your answer.

- Yes, it is within range and expected, because WR acts as the DHCP server and automatically assigns an IP address to Admin from its default DHCP pool (starting at .100).

Part 4 - Step 2 (f):

Why? (Web Browser will display a Request Timeout after a short time)

- Because Admin is still using its old IP address on the 192.168.0.0/24 network, but the router's LAN IP address has changed to 192.168.50.1 (network 192.168.50.0/24). Admin's old IP can no longer communicate with the router until it renews its IP address.

Part 4 - Step 2 (i):

What is the new IP address information for Admin?

- IP Address: 192.168.50.101

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.50.1

- DNS Server: 209.165.201.1

Part 4 - Step 2 (l):

Why? (Laptop1 connected to the AP instead of WR)

- Because the wireless signal (SSID: aCompany) that Laptop1 receives from the AP is stronger than the signal from WR at Laptop1's location, so the device automatically connects to the access point providing the strongest signal.